Title: Kinetic Data, Model, and Aspen Plus Information for Ammonia Synthesis with Fe Catalyst

## Aim

- To investigate the reaction mechanism and develop reliable kinetic models for ammonia synthesis over iron-based catalysts under industrial operating conditions.

- To investigate the kinetic performance of iron-based and promoted metal catalysts for ammonia synthesis and evaluate their applicability in catalytic reactor modeling and process simulation.

Objectives

- To study the reaction mechanism of ammonia synthesis on iron catalyst surfaces, particularly focusing on nitrogen adsorption and dissociation steps.

- To analyze the kinetic behavior of ammonia synthesis using Temkin-type and Temkin–Pyzhev kinetic models.

- To analyze the Arrhenius relationship between temperature and reaction rate constants.

- To study the ammonia synthesis kinetics using iron catalysts promoted with other metals such as potassium, ruthenium, or cobalt.

- To compare the catalytic performance of iron catalysts with alternative catalysts such as Ru-based systems.

- To analyze the effect of catalyst composition on reaction rate, activation energy, and reaction orders.

# Expected Outcomes

- Identification of reliable kinetic models describing ammonia synthesis over iron-based catalysts.

- Improved understanding of the reaction mechanism and rate-determining step in ammonia synthesis.

- Comparative evaluation of iron-based and promoted catalysts in terms of catalytic activity and kinetic performance.

- Development of a validated reactor model for ammonia synthesis using Aspen Plus.

### Chapter 1. Summary of Kinetic Data and Equation for Ammonia Synthesis

1.1. Ammonia synthesis rate over a wide operating range: from experiments to validated kinetic models. [1]

Ammonia synthesis is carried out industrially through the Haber–Bosch process, where nitrogen reacts with hydrogen over an iron-based catalyst to produce ammonia according to the reversible reaction:

N2+3H2⇌2NH3
This reaction is highly exothermic and equilibrium-limited, requiring high pressure and moderate temperature to achieve practical conversion. The kinetic behavior of ammonia synthesis is commonly described using Temkin-type kinetic models, which account for the influence of reactant and product partial pressures on the reaction rate.

The kinetic study was performed using a magnetite-based industrial iron catalyst in a fixed-bed reactor. Experiments were conducted over a temperature range of 548–773 K at a pressure of 90 bar. The feed gas mixture mainly consisted of hydrogen and nitrogen with a typical composition of 75 mol% H₂ and 25 mol% N₂, corresponding to an approximate H₂/N₂ ratio of 3, which is close to the stoichiometric requirement for ammonia formation. The catalytic reaction experiments were performed at a total gas flow rate of about 200 NmL min⁻¹, while 300 NmL min⁻¹ was used during partial pressure variation studies. The H₂/N₂ ratio was varied between 4:1 and 1:1 to examine the effect of feed composition on ammonia synthesis kinetics.

The reaction rate was analyzed using the Temkin-type kinetic model modified by Dyson and Simon, which expresses the ammonia synthesis rate as a function of the activities or partial pressures of nitrogen, hydrogen, and ammonia. The general Temkin rate expression is written as:

r=kaN2aH23aNH32α-kaNH32aH231-α
where r is the reaction rate, k is the rate constant, a represents the activity of each component, and α is the Temkin reaction order parameter. The parameter α reflects the influence of adsorption effects and typically lies between 0.5 and 0.75 for industrial iron catalysts.

The temperature dependence of the rate constant follows the Arrhenius equation:

k=k0exp⁡EaRT
From the kinetic analysis, the optimal Temkin parameter was determined as α = 0.654. The Arrhenius parameters obtained from experimental data include a pre-exponential factor k0=6.5×1013kmol m⁻³ h⁻¹ and an activation energy Ea=159.4kJ mol⁻¹. These values are consistent with previously reported kinetic data for ammonia synthesis over promoted iron catalysts.

The experimental results show that the ammonia synthesis rate increases with temperature due to enhanced reaction kinetics; however, at higher temperatures the conversion becomes limited by thermodynamic equilibrium. Additionally, higher hydrogen partial pressure can inhibit the reaction because hydrogen occupies active sites on the catalyst surface, reducing nitrogen adsorption. In contrast, increasing nitrogen partial pressure generally enhances ammonia synthesis rates.

Overall, the Temkin kinetic model with the above parameters provides a reliable description of ammonia synthesis over iron catalysts under high-pressure conditions and is widely used for reactor design, kinetic modeling, and process simulation of ammonia synthesis reactors.

Fig. 1. Dependence of identified rate constant k of Dysonand Simon’s rate equation on temperature for α=0.654.

  - A century of data: Thermodynamics and kinetics for ammonia synthesis on various commercial iron-based catalysts.  [2]

Ammonia is synthesized industrially through the Haber–Bosch reaction:

N2+3H2⇌2NH3
This reaction is exothermic (ΔH° ≈ −92 kJ mol⁻¹) and equilibrium-limited. Industrial ammonia synthesis is typically carried out over promoted iron catalysts derived from magnetite or wüstite precursors with promoters such as K2O, Al2O₃, MgO, SiO2, and CaO, which enhance catalytic activity and stability. Typical industrial operating conditions are 350–500 °C (623–773 K) and 100–300 bar.

The thermodynamic behavior of the gas mixture is described using a modified Soave–Redlich–Kwong (SRK) equation of state including a polarity correction factor for ammonia (ppol = −0.2368) to account for non-ideal behavior at high pressures. The equilibrium constant expressed using fugacity is:

Kf=PNH3PN20.5)PH20.5)ϕNH3ϕN20.5ϕH21.5
For process simulators such as Aspen Plus, an Arrhenius-type correlation for equilibrium is used:

Kf=1.3628×10-6exp⁡-5.1111×104RT
which is valid in the 300–550 °C temperature range.

The reaction mechanism follows a Langmuir–Hinshelwood adsorption model in which nitrogen and hydrogen adsorb on the catalyst surface and react through a sequence of hydrogenation steps forming NH*, NH₂*, and NH₃*. The dissociation of adsorbed nitrogen (N₂) into atomic nitrogen (N) is the rate-determining step** on the iron surface.

The classical Temkin–Pyzhev kinetic equation describing ammonia synthesis rate is:

rNH3=kNH3+PN2PH23PNH32α-kNH3-PNH32PH231-α
where α represents catalyst surface heterogeneity and typically ranges from 0.25–0.75.

A more detailed Langmuir–Hinshelwood kinetic model including adsorption effects is expressed as:

rNH3=bcatkNH3fN211Kf2fNH32fN2fH231fNH3KNfH21.5KHfH2-2
where bcat represents the relative catalytic activity, and KN and KH are adsorption constants.

The kinetic parameters follow the Arrhenius equation:

k=Ae-Ea/RT
Typical fitted values include a pre-exponential factor 2.199×1012and an activation parameter 1.683 × 10⁵, with α ≈ 0.27.

The kinetic model was validated using 11 datasets (1019 experimental data points) covering a wide operating range: temperature 251–550 °C, pressure 1–324 bar, H₂/N₂ ratio 0.33–8.5, and space velocity 1–1800 Nm³ kgcat⁻¹ h⁻¹.

For Aspen Plus simulation, the reactor is modeled as a plug flow reactor (RPlug) representing a packed catalytic bed. The governing mass balance is:

ρg0SVdwidz=Mw,iνirNH3
where SV is the space velocity, rNH3is the reaction rate, and wirepresents the component mass fraction.

Overall, the Temkin-based Langmuir–Hinshelwood kinetic model combined with the modified SRK thermodynamic model provides reliable kinetic parameters for implementing ammonia synthesis reactors in Aspen Plus simulations and industrial reactor design.

  - Reaction Mechanisms, Kinetics, and Improved Catalysts for Ammonia Synthesis from Hierarchical High Throughput Catalyst Design [3]

Ammonia synthesis is carried out industrially through the Haber–Bosch reaction:

N2+3H2⇌2NH3
(ΔH ≈ −92 kJ mol⁻¹) Industrial ammonia reactors typically operate at temperature 700–800 K and pressure 50–200 atm using promoted iron catalysts.

Reaction Mechanism and Kinetic Model

The catalytic reaction follows a Langmuir–Hinshelwood surface mechanism

The rate-controlling steps generally include N₂ adsorption/dissociation, NH₂ hydrogenation, and NH₃ desorption. Computational studies using Density Functional Theory (DFT) and kinetic Monte Carlo (kMC) simulations predicted a turnover frequency (TOF) of about 17.7 NH₃ molecules s⁻¹ per active site at T = 673 K and P(H2/N2/NH3) = 15/5/1 atm.

Kinetic Rate Expression

The reaction rate constant is described using transition state theory (TST):

k=kBThexp⁡ΔG‡kBT
where kBis Boltzmann constant, his Planck constant, Tis temperature, and ΔG‡is the activation free energy barrier.

Aspen Plus Modeling Data

For process simulation, ammonia synthesis is modeled in Aspen Plus using a plug flow reactor (RPlug) representing a packed-bed catalytic reactor.

Key simulation parameters:

Components: N₂, H₂, NH₃
Property method: Peng–Robinson (PR) or SRK
Operating temperature: 650–750 K
Pressure: 90–200 bar
Feed ratio: H₂/N₂ ≈ 3

The reactor mass balance along the catalyst bed is expressed as:

ρgSVdwidz=Mw,iνirNH3

where SVis space velocity, wiis species mass fraction, and rNH3is the ammonia formation rate.

  - Enhancing the photoactivation effect on alumina-modified α-iron for highly efficient photothermal catalytic ammonia synthesis [4].

Kinetic Study and Catalyst Behavior

The kinetic investigation was conducted using α-Fe catalysts modified with Al2O3, including Fe-0-Al, Fe-1%Al, and Fe-10%Al catalysts.

The apparent activation energy (Ea) for ammonia synthesis was obtained from Arrhenius plots:

- Fe-0-Al: 138.07 kJ mol⁻¹

- Fe-1%Al: 135.66 kJ mol⁻¹

- Fe-10%Al: 133.69 kJ mol⁻¹

Kinetic Model and Rate Equation

Ammonia synthesis follows a Langmuir–Hinshelwood heterogeneous catalytic mechanism, where nitrogen and hydrogen adsorb on the catalyst surface and react through stepwise hydrogenation. The simplified kinetic rate expression can be written as:

rNH3=kPN2αPH2βPNH3γ

where
rNH3= ammonia formation rate, k= rate constant, Pi= partial pressure of components,
α,β,γ= reaction orders.

The rate constant follows the Arrhenius equation:

k=Aexp⁡-EaRT
Aspen Plus Reactor Modeling Data

For Aspen Plus simulation, ammonia synthesis is modeled using a plug flow reactor (RPlug) representing a packed-bed catalytic reactor.

Components: N₂, H₂, NH₃
Property method: Peng–Robinson or SRK equation of state

Operating conditions:
Temperature: 673–773 K
Pressure: 20–25 MPa
Feed ratio: H2/N2 ≈ 3

Reactor parameters:
Catalyst loading: 0.02 g
GHSV: 180,000 mL g⁻¹ h⁻¹

The reactor mass balance along the catalyst bed is:

ρgSVdwidz=Mw,iνirNH3
where SVis space velocity, wiis species mass fraction, and rNH3is the ammonia formation rate.

  - Reaction Mechanism and Kinetics for Ammonia Synthesis on the Fe Surface [5]

Ammonia synthesis is carried out through the Haber–Bosch reaction:

N2+3H2⇌2NH3

This reaction is exothermic (ΔH ≈ −92 kJ mol⁻¹) and equilibrium-limited. Industrial ammonia production therefore requires high temperature (700–850 K) and high pressure (50–200 atm) over iron-based catalysts, particularly the Fe(111) surface, which provides the most active catalytic sites for nitrogen dissociation.

Computational studies using DFT and kinetic Monte Carlo (kMC) simulations indicate that the reaction rate is mainly controlled by N₂ dissociation, NH₂ hydrogenation, and NH₃ desorption. Under conditions of 673 K and 20 atm, the predicted turnover frequency (TOF) is about 17.7 NH₃ molecules s⁻¹ per (2 × 2) Fe surface site, which agrees with experimental values.

Kinetic Model and Rate Equation

The reaction rate constant follows transition state theory (TST):

k=kBThexp⁡( -1.57kBT)

The overall ammonia formation rate can be expressed as:

rNH3=kfN2fH23fNH32Keq
where firepresents fugacity and Keqis the equilibrium constant.

Aspen Plus Reactor Data

For Aspen Plus simulation, ammonia synthesis is modeled using a plug flow reactor (RPlug) representing a catalytic packed-bed reactor.

Key Aspen inputs:

Components: N2, H2, NH3
Property method: Peng–Robinson or SRK EOS
Temperature: 673–750 K
Pressure: 50–200 atm
Feed ratio: H2/N2 ≈ 3

### Chapter 2. Iron with other metal catalyst ammonia synthesis with kinetic study

  - Process simulation of ammonia synthesis over optimized Ru/C catalyst and multibed Fe + Ru configurations [6]

Ammonia synthesis is carried out through the Haber–Bosch reaction:

N2+3H2⇌2NH3
This reaction is exothermic and equilibrium limited, requiring high pressure and moderate temperature for industrial production. The kinetic behavior of ammonia synthesis in process simulations is typically modeled using the Langmuir–Hinshelwood–Hougen–Watson (LHHW) framework in Aspen Plus, which accounts for surface reactions and adsorption effects on catalysts.

Kinetic Study and Catalyst Models

The kinetic study considers two main catalyst systems: Ru/C catalysts and Fe-based catalysts (magnetite and wustite).

For Ru/C catalysts, the reaction kinetics are represented by a modified Temkin–LHHW model that includes adsorption inhibition effects of hydrogen and ammonia on the catalyst surface.

For Fe-based catalysts, including magnetite and wustite, simpler power-law kinetic models are used. These models describe the reaction rate using reactant activities without explicit adsorption terms in the denominator.

General LHHW Rate Expression (Aspen Plus)

The general kinetic model implemented in Aspen Plus follows the LHHW formulation:

r=k0e-Ea/RT)(Kforrfrvr-KrevpfpvpKiifivi)e
where
k0= pre-exponential factor,
Ea= activation energy,
fi= fugacity of species,
Kforand Krev= forward and reverse equilibrium constants.

Fe-Based Catalyst Kinetic Models

Wustite Model

dηdτ=kλ(q)(Keq)2aN2aH22.25aNH31.5-aNH30.75aH20.75
KM Iron Catalyst Model

dηdτ=k(Keq)2aN2aH21.5aNH3-aNH3aH21.5
These equations express the ammonia formation rate in terms of reactant activities and equilibrium effects.

Aspen Plus Reactor and Thermodynamic Data

For process simulation, the ammonia synthesis reactor is modeled using the RPlug reactor module to represent a packed-bed catalytic reactor operating under adiabatic or isothermal conditions.

Thermodynamic Property Methods

- Peng–Robinson (PR) EOS

- Redlich–Kwong (RK) EOS

- Redlich–Kwong–Soave (RKS-BM) for closed synthesis loops

Catalyst Properties

Catalyst density used for kinetic parameter correction:

- Ru/C: 0.59 g cm⁻³

- Magnetite: 2.8–2.9 g cm⁻³

- Wustite: 3.25 g cm⁻³

Activation Energies

- Ru/C catalyst: 23.0 kcal mol⁻¹

- Magnetite catalyst: 47.5 kcal mol⁻¹

- Wustite catalyst: 45.0 kcal mol⁻¹

Equilibrium Constant Correlation

In Aspen Plus the equilibrium constant is expressed as:

ln⁡(K)=A+BT+Cln⁡(T)+DT
Example forward coefficients:

- Ru/C: A = 7.19, B = −4609

- Wustite: A = −1.876, B = 9218, C = 2.69, D = −5.42

- Magnetite: A = −7.8, B = 9218, C = 2.88, D = −5.42

Adsorption equilibrium constants for Ru catalysts include:

ln⁡KH2=-56.9024/R+37656/RT
ln⁡KNH3=34.7272/R+29228/RT
Table 1. Thermodynamic and kinetic parameters for the reaction models according to the Aspen Plus© formulation (8), where the equilibrium constants are expanded as:

| Rate expression | Eq. (2) (Ru/C) |  |  |  | Eq. (5) (Wustite) |  |  |  | Eq. (5)b (Magnetite) |  |  | Eq. (6) (‘KM’ Iron-based) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stoichiometry | N2 |  | H2 | NH3 | N2 |  | H2 | NH3 | N2 |  | H2 | NH3 |  | N2 |  | H2 | NH3 |
|  | −0.5 |  | −1.5 | 1 | −0.5 |  | −1.5 | 1 | −0.5 |  | −1.5 | 1 |  | −0.5 |  | −1.5 | 1 |
| Kinetic | Ea |  | k0 |  | Ea |  | k0 |  | Ea |  | k0 |  |  | Ea |  | k0 |  |
|  | 23.0 |  | 426 |  | 45 |  | 7.47 × 108 |  | 47.5 |  | 3.23 × 109 |  |  | 40.76 |  | 1.72 × 108 |  |
| Rate expression |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Exponents | N2 |  | H2 | NH3 | N2 |  | H2 | NH3 | N2 | H2 |  | NH3 |  | N2 |  | H2 | NH3 |
| Forward (νr) | 0.5 |  | 0.375 | −0.25 | 1 |  | 2.25 | −1.5 | 1 | 2.25 |  | −1.5 |  | 1 |  | 1.5 | −1 |
| Reverse (νp) | 0 |  | −1.125 | 0.75 | 0 |  | −0.75 | 0.5 | 0 | −0.75 |  | 0.5 |  | 0 |  | −1.5 | 1 |
| Coefficients | A | B | C | D | A | B | C | D | A | B |  | C | D | A | B | C | D |
| Term 1 (Kfor) | −7.19 | 0 | 0 | 0 | −7.8 | 9218 | −5.42 | 7.8 × 10−4 | −7.8 | 9218 |  | −5.42 | 7.8 × 10−4 | −4.9 | 9218 | −5.42 | 7.8 × 10−4 |
| Term 2 (Krev) | −1.876 | −4609 | 2.69 | 1.27 × 10−4 | 2.88 | 0 | 0 | 0 | 2.88 | 0 |  | 0 | 0 | 5.76 | 0 | 0 | 0 |
| Adsorption |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Exponents | N2 |  | H2 | NH3 | na |  |  |  | na |  |  |  |  | na |  |  |  |
| Term 1 | 0 |  | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Term 2 | 0 |  | 0.3 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Term 3 | 0 |  | 0 | 0.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coefficients | A | B | C | D |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Term 1 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Term 2 | −10.3 | 4529 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Term 3 | −6.48 | 3523 | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |

  - Reaction kinetics for ammonia synthesis using ruthenium and iron based catalysts under low temperature and pressure conditions [7]

Kinetic Study

The study evaluates the kinetics of ammonia (NH3) synthesis over iron (Fe) and Ru/CeO₂ catalysts under relatively mild operating conditions suitable for Power-to-Ammonia systems. Experimental investigations were carried out in the temperature range of 350–450 °C (623–723 K) and pressure range of 10–80 bar. The reaction mechanism follows the classical Haber–Bosch reaction:

N2+3H2⇌2NH3
The kinetic analysis is based on the Temkin–Pyzhev model, which assumes that the dissociative adsorption of nitrogen (N₂) on the catalyst surface is the rate-determining step. The model is modified to include adsorption inhibition effects of ammonia and hydrogen, which are important under practical reaction conditions.

Kinetic Model and Rate Equation

For the Fe catalyst, the Temkin-type rate equation is extended by including an adsorption term for ammonia in the denominator:

rNH3=k2aN20.5KeqaH21.5aNH3αaNH3aH21.51-α1+K3aNH3f3
For the Ru/CeO₂ catalyst, the kinetic model includes additional adsorption terms accounting for strong hydrogen inhibition, which is particularly significant at lower temperatures.

The temperature dependence of the kinetic parameters follows the Arrhenius equation:

k2=k2,0exp⁡EART

The adsorption equilibrium constants are determined using the van’t Hoff equation:

Ki=exp⁡-ΔHads,iRT+ΔSads,iR
where
k2,0= pre-exponential factor,
EA= activation energy,
ΔHads= adsorption enthalpy,
ΔSads= adsorption entropy.

Aspen Plus Implementation Data

For simulation in Aspen Plus, gas-phase activities (ai) are calculated using the Soave–Redlich–Kwong (SRK) equation of state.

Fe Catalyst Parameters

- Pre-exponential factor k2,0: 3.35 × 10⁹ mol s⁻¹ kg⁻¹

- Activation energy EA: 1.44 × 10⁵ J mol⁻¹

- Reaction order α: 0.5

- NH₃ adsorption enthalpy ΔH3,ads: −1.56 × 10⁴ J mol⁻¹

- NH₃ adsorption entropy ΔS3,ads: −24.4 J mol⁻¹ K⁻¹

- Catalyst bulk density ρb: 2500 kg m⁻³

Ru/CeO₂ Catalyst Parameters

- Pre-exponential factor k2,0: 3.49 × 10⁸ mol s⁻¹ kg⁻¹

- Activation energy EA: 1.06 × 10⁵ J mol⁻¹

- Reaction order α: 0.25

- H₂ adsorption enthalpy ΔH2,ads: −8.71 × 10⁴ J mol⁻¹

- H₂ adsorption entropy ΔS2,ads: −80.1 J mol⁻¹ K⁻¹

- Catalyst bulk density ρb: 2100 kg m⁻³

  - Economies of scale in ammonia synthesis loops embedded with iron- and ruthenium-based catalysts [8]

Kinetic Study

The study analyzes the economies of scale in ammonia synthesis loops by comparing conventional iron-based (Fe) catalysts with ruthenium-based (Ru/C) catalysts across plant capacities ranging from 0.1 to 500 tonne/day. The objective is to evaluate catalytic performance and process economics using detailed kinetic models integrated into Aspen Plus simulations. The ammonia synthesis reaction follows the Haber–Bosch reaction:

N2+3H2⇌2NH3

Different kinetic models are adopted to represent the distinct poisoning mechanisms of the catalysts. Fe catalysts are mainly affected by ammonia poisoning, whereas Ru/C catalysts are strongly influenced by hydrogen poisoning. Therefore, separate kinetic expressions are used for each catalyst to accurately describe their catalytic behavior.

Kinetic Models and Rate Equations

Fe-based Catalyst Model (Temkin Equation)

The reaction kinetics for Fe catalysts are described using the Temkin rate expression, which incorporates equilibrium and activity effects:

rNH3=krKa2aN2aH2)3aNH3)2αaNH3)2aH2)31-α
where
rNH3= ammonia formation rate,
ai= activity of component i,
Ka= equilibrium constant,
α= reaction order parameter.

The reverse reaction kinetic constant is expressed as:

kr=1.7698×1015exp⁡40765RcT
This model effectively describes Fe catalysts where ammonia adsorption inhibits the reaction rate.

Ru/C Catalyst Model (Modified Temkin–LHHW Equation)

For Ru/C catalysts, the kinetic expression is modified using the Langmuir–Hinshelwood–Hougen–Watson (LHHW) approach to account for hydrogen poisoning effects:

rN2=kf(aN2)0.5aH2)0.375aNH3)0.25-1KaaNH3)0.75aH2)1.1251+KH2(aH2)0.3+KNH3(aNH3)0.2
The temperature dependence of the forward kinetic constant is:

kf=9.02×108exp⁡23000RcT
Adsorption equilibrium constants are given by:

ln⁡KH2=-56.9024R+37656RT
ln⁡KNH3=34.7272R+29228RT
Aspen Plus Implementation Data

The kinetic models were implemented in Aspen Plus using a plug flow reactor (RPlug) with user-defined Fortran subroutines under adiabatic conditions.

Key simulation specifications include:

Reactor model: Packed-bed RPlug reactor
Feed composition: H2/N2=3
Thermodynamic model: Activities calculated using fugacity coefficients (ϕi) derived from correlations developed by Cooper, Shaw, and Newton.

Catalyst parameters used in simulation:

Fe Catalyst

- Reaction order parameter (α): 0.5

- Kinetic constant kr: 1.7698×1015exp⁡(-40765/RcT)

- Bulk density: 2.8 g cm⁻³

Ru/C Catalyst

- Kinetic constant kf: 9.02×108exp⁡(-23000/RcT)

- Hydrogen adsorption constant KH2: defined by van’t Hoff correlation

- Ammonia adsorption constant KNH3: defined by van’t Hoff correlation

- Bulk density: 0.8 g cm⁻³

  - Catalytic Behavior of K-doped Fe/MgO Catalysts for Ammonia Synthesis Under Mild Reaction Conditions [9]

Kinetic Study

The study examines the kinetics of ammonia synthesis over a potassium-promoted Fe/MgO catalyst (Fe/K/MgO) under relatively mild operating conditions of 200–400 °C and 0.1–3.0 MPa. Potassium doping significantly enhances catalytic performance by modifying the electronic properties of the Fe surface. As a result, the apparent activation energy decreases and the catalytic activity increases, particularly at temperatures above 300 °C.

Kinetic Model and Rate Equation

The kinetics are described using a Langmuir-type rate expression, which considers adsorption effects of reactants and products on catalyst active sites. The fundamental rate equation is expressed as:

r=kPN21K1PNH3PH2-(1.5-0.5x)K2PH20.5-2
where

r= ammonia synthesis rate,
k= rate constant for nitrogen adsorption,
Pi= partial pressure of species i,
K1and K2= adsorption equilibrium constants for NH3and H2, respectively,
x= reaction parameter related to hydrogenation steps.

To simplify kinetic analysis, the rate equation can be transformed into a power-law form using retardation indices that describe surface adsorption effects:

r=k'PN2PH2(3-x)y-zPNH3-2y
where

k'=kK1-2yK2-2z

and

y= retardation index for adsorbed NHxspecies,
z= retardation index for adsorbed hydrogen.

Aspen Plus Implementation Data

Although a detailed Aspen Plus flowsheet is not explicitly provided, the experimental kinetic parameters reported in the study allow implementation of the Fe/K(3)/MgO catalyst kinetics in process simulation tools such as Aspen Plus.

Key parameters required for reactor modeling include:

Operating temperature range: 200–400 °C

Operating pressure range: 0.1–3.0 MPa

Reaction orders at 350 °C:

- N2: 1.00

- H2: 2.30

- NH3: −1.53

Apparent activation energy:

- 96.5 kJ mol⁻¹ (200–300 °C range)

- 32.3 kJ mol⁻¹ (300–400 °C range)

Retardation indices (350 °C, x=0)

- y=0.77

- z=-0.01

Turnover frequency (TOF):

- 0.098 s⁻¹ at 350 °C and 1.0 MPa

  - Kinetic Studies of Ammonia Formation over Fe and Mo Containing HZSM-5 Catalysts [10]

Kinetic Study

The study investigates the kinetics of ammonia synthesis from nitrogen and hydrogen over Fe–Mo catalysts supported on HZSM-5 zeolite. Experiments were conducted at atmospheric pressure within a temperature range of 575–773 K to evaluate the catalytic activity and synergistic effects between iron (Fe) and molybdenum (Mo) loadings.

The ammonia synthesis reaction follows the well-known Haber reaction:

N2+3H2→2NH3

Kinetic Model and Rate Relationship

Unlike classical ammonia synthesis studies, the document does not provide a formal mechanistic kinetic equation such as Temkin–Pyzhev or Langmuir–Hinshelwood models. Instead, the reaction kinetics are described empirically based on experimental observations.

The ammonia formation rate can be expressed conceptually as a function of temperature and reactant flow rate:

rNH3=f(T,PN2,PH2)
where

rNH3= ammonia formation rate,
T= reaction temperature,
PN2and PH2= partial pressures of nitrogen and hydrogen.

The results indicate that the rate is positively correlated with temperature and dependent on hydrogen concentration, with an optimum flow rate range.

Aspen Plus Implementation Data

Although the study does not provide full kinetic parameters such as activation energy, adsorption constants, or pre-exponential factors, the experimental data can still be used to develop a custom kinetic model in Aspen Plus through regression or empirical fitting.

Key parameters useful for process simulation include:

Operating Conditions

- Temperature range: 573–773 K

- Pressure: 1 atm (atmospheric pressure)

Optimal Catalyst Composition

- Mo (18.8%) + Fe (2.5%) supported on HZSM-5

Surface Properties

- Catalyst surface area: 251.1 – 384.5 m² g⁻¹ depending on metal loading

Feed Composition

- Optimal molar ratio: H₂/N₂ = 3 : 1

Reactant Flow Rates (at STP)

- H2=24.68×10-7mol s⁻¹

- N2=7.43×10-7mol s⁻¹

  - New efficient catalyst for ammonia synthesis: barium-promoted cobalt on carbon [11]

Kinetic Model and Equation

The reaction rate is represented using a power-law model:

rNH3=kexp⁡EARTPNH3αPN2βPH2γ

Where:

rNH3: ammonia formation rate

k: pre-exponential factor

EA: activation energy (J/mol)

Pi: partial pressures

α,β,γ: reaction orders

This model is valid under low conversion conditions, as equilibrium limitations become significant at high ammonia concentrations.

Kinetic Parameters (Ba₀.₈Co₁.₀/C)

Activation Energy: 103 kJ/mol

Reaction Orders (400 °C):

αNH3=-1.1

βN2=0.9

γH2=0.9

Temperature Dependence:

α=-0.81to -1.07(320–400 °C)

γ=0.7to 2.1 (320–440 °C)

Commercial Fe Catalyst (KM1) for Comparison

Activation Energy: 70 kJ/mol

Reaction Orders (400 °C):

α=-1.5, β=0.9, γ=2.2

Aspen Plus Implementation Data

Reactor Type: RPlug (preferred) or RCSTR
Reaction Type: Power-law kinetics

Required Inputs:

Activation Energy: 103000 J/mol

Reaction Orders: NH₃ = −1.1, N₂ = 0.9, H₂ = 0.9

Pre-exponential factor (k): estimated via regression

Operating Conditions:

Temperature: 320–440 °C

Pressure: 2–50 bar

Feed ratio: N2:H2=1:3

### References

1. 	Nadiri S, Attari Moghaddam A, Folke J, et al. Ammonia Synthesis Rate Over a Wide Operating Range: From Experiments to Validated Kinetic Models. ChemCatChem. 16(23) (2024).

2. 	Keestra H, Slotboom Y, Rouwenhorst KHR, Brilman DWF. A century of data: Thermodynamics and kinetics for ammonia synthesis on various commercial iron-based catalysts. Chem. Eng. J. [Internet]. 518(May), 164593 (2025). Available from: https://doi.org/10.1016/j.cej.2025.164593.

3. 	Fuller J, An Q, Fortunelli A, Goddard WA. Reaction Mechanisms, Kinetics, and Improved Catalysts for Ammonia Synthesis from Hierarchical High Throughput Catalyst Design. Acc. Chem. Res. 55(8), 1124–1134 (2022).

4. 	Ding Y, Ma H, Yang Y, et al. Enhancing the photoactivation effect on alumina-modified α‑iron for highly efficient photothermal catalytic ammonia synthesis. Chem. Eng. J. [Internet]. 525(October), 170180 (2025). Available from: https://doi.org/10.1016/j.cej.2025.170180.

5. 	Qian J, An Q, Fortunelli A, Nielsen RJ, Goddard WA. Reaction Mechanism and Kinetics for Ammonia Synthesis on the Fe(111) Surface. J. Am. Chem. Soc. 140(20), 6288–6297 (2018).

6. 	Tripodi A, Compagnoni M, Bahadori E, Rossetti I. Process simulation of ammonia synthesis over optimized Ru/C catalyst and multibed Fe + Ru configurations. J. Ind. Eng. Chem.66, 176–186 (2018).

7. 	Cholewa T, Steinbach B, Heim C, et al. Reaction kinetics for ammonia synthesis using ruthenium and iron based catalysts under low temperature and pressure conditions. Sustain. Energy Fuels. 8(10), 2245–2255 (2024).

8. 	Yoshida M, Ogawa T, Imamura Y, Ishihara KN. Economies of scale in ammonia synthesis loops embedded with iron- and ruthenium-based catalysts. Int. J. Hydrogen Energy [Internet]. 46(57), 28840–28854 (2021). Available from: https://doi.org/10.1016/j.ijhydene.2020.12.081.

9. 	Era K, Sato K, Miyahara S ichiro, et al. Catalytic Behavior of K-doped Fe/MgO Catalysts for Ammonia Synthesis Under Mild Reaction Conditions. ChemSusChem. 16(22), 1–9 (2023).

10. 	Unde RB, Gaikwad AG. Kinetic studies of ammonia formation over Fe and Mo containing HZSM-5 catalysts. Chem. Biochem. Eng. Q. 21(2), 139–144 (2007).

11. 	Hagen S, Barfod R, Fehrmann R, et al. New efficient catalyst for ammonia synthesis: Barium-promoted cobalt on carbon. Chem. Commun. 11, 1206–1207 (2002).